# DEMOLITION DERBY

# COMPETITOR TOOLBOX TALK

Revised Competitor Briefing (effective 1 January 2026)

AgShows NSW – Mandatory Safety Briefing for Drivers & Pit Crew

## INTRODUCTION

Good [morning/afternoon]. I am [insert name], the Chief Steward/Scrutineer for today’s Derby. This briefing is mandatory. It summarises the rules, safety requirements, and ring procedures. Failure to comply means you cannot compete.

## 1. Event Risks

Demolition Derbies are high-risk. Serious injury can occur.
If you have concerns about your vehicle, gear, arena conditions, or another driver, tell an official before the event starts. If it cannot be made safe, you must withdraw.

## 2. Mandatory Rules & Requirements

This event is run under:

- AgShows NSW Competitor Rules (2026)

- Ring Control Guidelines

- Code of Conduct

- Waiver & Conditions of Entry

- Drug & Alcohol Policy: 0.00 BAC and negative drug test if required

If you cannot meet these standards, you cannot compete.

## 3. Driver Safety Gear

You must wear:

- Full-face helmet + visor/goggles

- Neck brace / SFI foam collar

- Cotton drill long-sleeve overalls (no nylon)

- Leather gloves & enclosed footwear (no thongs, sneakers)
Minimum age: 17 years, P-plate licence, guardian waiver if under 18.

## 4. Vehicle Compliance

Your vehicle must pass full scrutineering, including:

- Required driver safety bars and driver-door bracing

- Interior stripped of glass, plastics, linings and loose objects

- Approved fuel tank + secure mounting

- Kill switch clearly marked

- Battery secured + covered

- Padding on all barwork

Tyres: standard road tyres only.
Vehicle eligibility: no 4WDs, no full-chassis vehicles, no EV/hybrid, V8 only with insurer approval.
Non-compliant vehicles will be excluded.

## 5. Surrender Flag & In-Ring Conduct

- All cars must start with the white surrender flag raised.

- If out or immobile: snap the flag off and stay in the vehicle.

- Do not exit unless instructed or in immediate danger.

- No striking surrendered, inactive, or boundary-out vehicles.

- No driver-door hits — automatic disqualification.

## 6. Control Car, Lights & Signals

Obey all officials immediately:

- Green light – continue

- Red flashing lights – stop immediately

- Spotlight – warning or disqualification

## 7. Movement, Boundaries & Elimination

- Use reverse gear only, except to dislodge your vehicle (max 2 car lengths).

- If you leave the boundary, re-enter slowly and near your exit point.

- Vehicles inactive for 45 seconds (or local rule) are out.
The Judge’s decision is final.

## 8. Emergencies

For fire, rollover or medical incidents:

- Control Car will enter with red lights

- All vehicles must stop immediately

- Do not leave your vehicle unless directed
Rolled vehicles are not to be hit under any circumstances.

## 9. Final Check

If you are unsure about:

- your vehicle

- your gear

- safety concerns

- ring conditions

…raise it now.

## Closing

Follow instructions, stay alert, and respect the rules so we can run a safe, compliant and enjoyable event.
Any questions?
Thank you, and stay safe.